| # SUPPLIER: | * * * |  | ### PROGRAM: | Hub Bearing |  |
| --- | --- | --- | --- | --- | --- |
| MFC. LOCATION: | * * * |  | SUPPLIER CODE: | * * * |  |
| PART NUMBER (S): | * * * |  | PART NAME (S): | * * * |  |
| CHANGE LEVEL (S): | * * * |  | DATE OF VISIT / NUMBER: |  | YYYY-MM-DD-** |

## COMPLIANCE REPORT

-	As part of the PSO, the actual measurement process must be witnessed and / or verified by the PSO Team.

- A minimum of 3 characteristics is to be checked. For any given part, assembly, or process, the number of

Characteristics selected depend upon the complexity.

-	A minimum of 30 randomly selected samples shall be checked.

-	Variable data is preferred but attribute data is acceptable.

-	Minimum value for Pp and Ppk should be 1.67 or greater.

-	For attribute data, a 30 pieces sample with one or more non-conformances is unacceptable.

| #### SUMMARY EXAMPLE |  |  |  |
| --- | --- | --- | --- |
| Characteristic / spec | Sample size | PP / PPK | ACC/UNACC |
| Length (13.6 ~ 17.5 mm) | 30 | 1.3/1.1 | UNACC |
| Width (11.2 ~ 11.8 mm) | 30 | 1.4/1.2 | UNACC |
| Hole Size (12.0  0.02 mm) | 30 | 1.8/1.7 | ACC |

| #### DATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Characteristic / spec |  | Sample size | PP / PPK | ACC/UNACC |
| Outer Ring Rough Grinding: | De | 30 | 0.516/0.316 | UNACC |
|  | VDep | 30 | 0.953/0.564 | UNACC |
|  | Se | 30 | 1.136/0.536 | UNACC |
| Inner Ring Rough Grinding: | di | 30 | 1.071/0.571 | UNACC |
|  | Vdip | 30 | 0.735/0.269 | UNACC |
|  | Si | 30 | 0.964/0.836 | UNACC |
| Inner Ring Finish Grinding: | di | 30 | 0.819/0.410 | UNACC |
|  | Vdip | 30 | 1.162/0.594 | UNACC |
|  | Si | 30 | 1.017/0.759 | UNACC |

| KEY: | NOTE: Attach supporting detailed data and chart |
| --- | --- |
| ACC = Acceptable | GOAL: |
| LSL = Lower Specification Limit | PP, PPK  1.67 |
| PP = (USL - LSL)  6s | The difference between PP and PPK should be as small as possible. |
| USL−x÷3s or x−LSL÷3s | Ideally PP - PPK = 0 |
| PPK = Smaller Of… |  |
| x=USL = Upper Specification Limit Mean Of The Sample Measurements |  |